BM64 I2S/SPK out Configuration

Aux-in/MIC in can be configured in BM64 in 4 possible combinations

- Analog in and Analog out

Please choose the following in UI tool

And following in DSP tool

- Analog in and I2S out

Please choose following in UI tool

And following in DSP Tool

- I2S in and I2S out

Please choose the following in UI tool

And choose following in DSP Tool

- I2S in and Analog out

For this configuration choose the following in the UI tool

And chose the following in DSP Tool

Create *.ipf file and program it into BM64. Since this configuration is not supported in UI/DSP tools, hence the following changes needs to be made in EEPROM location

0x1FC = 0x4A

0x344 = 0x0

0xAA0 = 0x3

0xAB4 = 0x3

Note 1: These address is specific to MSPK EEPROM table. DSPK will have different address.

Note2: When I2S_in is selected then it is advisable to use I2S slave mode.

Testing

First two configurations can be tested easily using one BM64 EVB.

- Please program attached MCHP_Multi_Analog1.ipf for testing Analog-in/Analog-out configuration

- Please Program EEPROM with MCHP_MULTI_03.ipf for Analog-in/I2S-out configuration.

- For testing I2S-in/I2S-Out configuration, two BM64 EVB is needed. Program Board 1 with MCHP_Multi_03.ipf. Program with Board 2 with MCHP_Multi_I2Sin_I2Sout.ipf

Make the following connections between Jumper J6 of Board 1 and Board 2

BOARD 1                         BOARD 2

RFSO(3)-------------------RFSO(3)

SCLK0(7)------------------SCLK0(7)

GND(5)--------------------GND(5)

DT0(9)---------------------DR0(1)

Pair and connect Board with a cell phone and play music. Power on board 2 and connect a speaker with R+/R-. Also insert a cable in line-in. This is to enable line-in detect code in MCU.

Music will be heard on connected speaker of Board 2.

- For testing I2S-in/Analog-Out configuration, two BM64 EVB is needed. Program Board 1 with MCHP_Multi_03.ipf. Program with Board 2 with MCHP_Multi_I2Sin_Analogout.ipf. Make the following changes in EEPROM of the board 2 using MicrochipBTUtil.7Z.

0x1FC = 0x4A

0x344 = 0x0

0xAA0 = 0x3

0xAB4 = 0x3

Make the following connections between Jumper J6 of Board 1 and Board 2

BOARD 1                         BOARD 2

RFSO(3)-------------------RFSO(3)

SCLK0(7)------------------SCLK0(7)

GND(5)--------------------GND(5)

DT0(9)---------------------DR0(1)

Pair and connect Board with a cell phone and play music. Power on board 2 and connect a speaker with SPK. Also insert a cable in line-in. This is to enable line-in detect code in MCU.

Music will be heard on connected speaker of Board 2.